# VORTEX: Voice-Operated Robotic Telemetry & Execution eXchange

# Technical Specification, Circuit Interfacing, Firmware & Build Manual

Author: Veerla Sagar
Project Repository: esp32-vortex-iot on GitHub

# 1. Executive Summary & Project Overview

VORTEX is an integrated multi-tier embedded robotics and smart home automation ecosystem. It unifies offline speech recognition, 4WD robotic locomotion, distributed IoT appliance switching, and emotive visual feedback into a decoupled hardware platform.

The system is designed with three core design tenets:

- Zero-Latency Offline Speech AI: Offline natural language parsing via the VC-02 module over hardware UART with zero cloud dependence and total operational privacy.

- Decoupled Asynchronous Processing: An ESP32 master handles speech decoding and motor driving, while an ESP8266 slave handles AC/DC relay switching and climate telemetry over Blynk IoT Cloud, and an Arduino Uno drives real-time OLED facial animations.

- Resilient Network Provisioning: Autonomous captive portal management using WiFiManager eliminates hardcoded credentials.

# 2. Distributed System Architecture

The system functions across three dedicated microcontroller nodes:

- Master Node (ESP32 Dev Module): Interfaced with the VC-02 voice module via UART1 (GPIO 16/17). Drives the 4WD chassis via an L298N H-bridge driver with PWM speed modulation, triggers audio playback via DFPlayer Mini, and sends single-byte facial animation states to the Arduino Uno.

- Remote Appliance Node (NodeMCU ESP8266): Connects to the local Wi-Fi network and Blynk Cloud. Controls a 4-channel relay module using active-LOW logic and reads ambient temperature and humidity every 2 seconds via a DHT22 sensor.

- Visual Feedback Core (Arduino Uno): Dedicated I2C controller for a 0.96-inch SSD1306 OLED screen (128x64) rendering emotive animated eye states (Idle, Happy, Thinking, Sleep, Wake).

# 3. Bill of Materials (BOM) & Component Specifications

- ESP32 Dev Module (30-Pin / 38-Pin): Master controller with dual-core 240 MHz MCU, Wi-Fi & Bluetooth.

- NodeMCU ESP8266 (ESP-12E): Secondary Wi-Fi controller for IoT relay switching.

- Arduino Uno R3: ATmega328P microcontroller for dedicated OLED animation rendering.

- VC-02 Speech Recognition Module (Ai-Thinker): Offline speech processing module.

- DFPlayer Mini MP3 Module & 8-Ohm Speaker: Voice response audio synthesizer with MicroSD storage.

- 0.96" SSD1306 I2C OLED Display (128x64): Graphic display for emotive expressions.

- L298N Dual H-Bridge Motor Driver: 2A per channel motor driver with onboard 5V regulator.

- 4WD Robot Chassis Kit: 4 DC gear motors, wheels, and acrylic chassis plate.

- 4-Channel 5V Relay Module: Optocoupler-isolated relays for 230V AC / 30V DC loads.

- DHT22 (AM2302) Sensor: Precision digital temperature and humidity sensor.

- Power Supply: 3S 11.1V/12V Lithium-ion battery pack and 5V step-down regulator.

# 4. Hardware Wiring & Pinout Tables

## 4.1. Master ESP32 Connections

| Target Device | Device Pin | ESP32 GPIO | Function / Signal |
| --- | --- | --- | --- |
| VC-02 Module | TX | GPIO 16 (RX2) | Voice Command Input |
| VC-02 Module | RX | GPIO 17 (TX2) | Configuration / Feedback |
| Arduino Uno | RX (Pin 0) | GPIO 2 (TX2) | Serial Expression Triggers |
| L298N Driver | IN1 | GPIO 27 | Left Motor Forward |
| L298N Driver | IN2 | GPIO 26 | Left Motor Backward |
| L298N Driver | IN3 | GPIO 25 | Right Motor Forward |
| L298N Driver | IN4 | GPIO 33 | Right Motor Backward |
| L298N Driver | ENA | GPIO 13 | Left Motor PWM Speed |
| L298N Driver | ENB | GPIO 12 | Right Motor PWM Speed |

## 4.2. Remote Slave ESP8266 Connections

| Target Device | Device Pin | NodeMCU Pin | GPIO | Function / Signal |
| --- | --- | --- | --- | --- |
| Relay 1 (Light) | IN1 | D1 | GPIO 5 | Active-LOW Switching |
| Relay 2 (Fan) | IN2 | D2 | GPIO 4 | Active-LOW Switching |
| Relay 3 (Water Pump) | IN3 | D5 | GPIO 14 | Active-LOW Switching |
| Relay 4 (Socket) | IN4 | D6 | GPIO 12 | Active-LOW Switching |
| DHT22 Sensor | DATA | D7 | GPIO 13 | Digital Telemetry Signal |

## 4.3. Arduino Uno OLED Connections

| OLED Display Pin | Arduino Uno Pin | Signal Description |
| --- | --- | --- |
| VCC | 5V | Logic Power |
| GND | GND | Ground |
| SDA | A4 (SDA) | I2C Data Line |
| SCL | A5 (SCL) | I2C Clock Line |
| Serial RX (Pin 0) | ESP32 GPIO 2 | Animation Command Trigger |

# 5. Voice Command & System Mapping Reference

| ID | Voice Command Phrase | Serial Code | Subsystem Action | OLED Animation State |
| --- | --- | --- | --- | --- |
| 1 | Move Forward | 'F' | 4WD Motors Forward | Happy / Forward |
| 2 | Move Backward | 'B' | 4WD Motors Reverse | Sad / Reverse |
| 3 | Turn Left | 'L' | 4WD Motors Spin Left | Look Left |
| 4 | Turn Right | 'R' | 4WD Motors Spin Right | Look Right |
| 5 | Stop Now | 'S' | All Motors Halted | Neutral / Idle |
| 10 | Happy Expression | 'H' | - | Happy Curved Eyes |
| 11 | Thinking Mode | 'T' | - | Thinking Inquiry Rings |
| 12 | Sleep Mode | 'Z' | - | Sleep Bars |
| 13 | Idle Mode | 'I' | - | Standard Rounded Gaze |
| 14 | Light ON | 'H' | Slave Relay 1 ON (V1=0) | Happy Eyes |
| 15 | Light OFF | - | Slave Relay 1 OFF (V1=1) | Neutral Eyes |
| 16 | Fan ON | 'H' | Slave Relay 2 ON (V2=0) | Happy Eyes |
| 17 | Fan OFF | - | Slave Relay 2 OFF (V2=1) | Neutral Eyes |
| 18 | Motor / Pump ON | - | Slave Relay 3 ON (V3=0) | Syncing |
| 19 | Motor / Pump OFF | - | Slave Relay 3 OFF (V3=1) | Neutral Eyes |
| 20 | Socket ON | - | Slave Relay 4 ON (V4=0) | Syncing |
| 21 | Socket OFF | - | Slave Relay 4 OFF (V4=1) | Neutral Eyes |
| 23 | Reset System | - | ESP.restart() | Boot Screen |
| 24 | Wake Up | 'W' | System Active | Expanding Eye Blocks |

## 6.3. OLED Facial Expressions Firmware (Arduino Uno)

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, -1);

void showBoot() {

display.clearDisplay();

display.setTextSize(2);

display.setTextColor(WHITE);

display.setCursor(20, 25);

display.print("VORTEX");

display.display();

}

void drawIdle() {

display.clearDisplay();

display.fillRoundRect(35, 22, 20, 20, 10, WHITE);

display.fillRoundRect(73, 22, 20, 20, 10, WHITE);

display.display();

}

void drawHappy() {

display.clearDisplay();

display.fillCircle(44, 32, 15, WHITE);

display.fillCircle(84, 32, 15, WHITE);

display.fillRect(0, 32, 128, 32, BLACK);

display.display();

}

void drawThinking() {

display.clearDisplay();

display.drawCircle(44, 32, 12, WHITE);

display.drawCircle(84, 32, 12, WHITE);

display.display();

}

void drawSleep() {

display.clearDisplay();

display.fillRect(35, 35, 20, 4, WHITE);

display.fillRect(73, 35, 20, 4, WHITE);

display.display();

}

void drawWake() {

display.clearDisplay();

display.fillRoundRect(30, 15, 30, 30, 5, WHITE);

display.fillRoundRect(68, 15, 30, 30, 5, WHITE);

display.display();

}

void setup() {

Serial.begin(9600);

display.begin(SSD1306_SWITCHCAPVCC, 0x3C);

display.clearDisplay();

showBoot();

delay(1500);

drawIdle();

}

void loop() {

if (Serial.available() > 0) {

char cmd = Serial.read();

switch (cmd) {

case 'H': drawHappy(); break;

case 'T': drawThinking(); break;

case 'Z': drawSleep(); break;

case 'W': drawWake(); break;

default: drawIdle(); break;

}

}

}

# 7. Step-by-Step Build & Commissioning Guide

- Circuit Assembly: Establish a solid common ground across all boards. Wire the VC-02, DFPlayer, L298N, OLED, relays, and DHT22 per Section 4.

- Library Installation: Install WiFiManager, Blynk, Adafruit_SSD1306, Adafruit_GFX, DHT, and DFRobotDFPlayerMini in Arduino IDE.

- Flashing: Insert Blynk template and auth tokens into the source sketches, select the corresponding COM ports, and upload the code to each board. (Note: Disconnect Arduino Uno Pin 0 (RX) during upload).

- Network Provisioning: Power up the system. Connect a smartphone to VORTEX_MASTER_AP and configure the Wi-Fi credentials. Repeat for VORTEX_SLAVE_AP.

- System Validation: Speak commands and verify simultaneous locomotion, relay actuation, OLED emotional changes, and live Blynk cloud telemetry.